/*

* ============================================================

*   HOME AUTOMATION WITH BLUETOOTH

*   Arduino Uno + HC-05 Bluetooth Module + Relay Module

* ============================================================

*   Author  : Internship Project

*   Date    : 2025

*   Version : 1.0

*

*   DESCRIPTION:

*   This sketch receives single-character commands via Bluetooth

*   (HC-05 module on Software Serial pins 10/11) and toggles

*   up to 4 relay channels, each controlling an AC load.

*

*   COMMAND TABLE:

*   '1' → Relay 1 ON   (Device 1 – Bulb/Light)

*   '2' → Relay 2 ON   (Device 2 – Fan)

*   '3' → Relay 3 ON   (Device 3 – Socket/Appliance)

*   '4' → Relay 4 ON   (Device 4 – Spare)

*   'a' → Relay 1 OFF

*   'b' → Relay 2 OFF

*   'c' → Relay 3 OFF

*   'd' → Relay 4 OFF

*   'A' → ALL Relays ON

*   'X' → ALL Relays OFF

*   'S' → Status report (which relays are ON)

* ============================================================

*/

#include <SoftwareSerial.h>

// ── Pin Definitions ──────────────────────────────────────────

// HC-05: RX pin of module → D11 (TX of Arduino SW serial)

//         TX pin of module → D10 (RX of Arduino SW serial)

SoftwareSerial BTSerial(10, 11);  // (RX, TX) from Arduino's perspective

// Relay pins (active-LOW for most relay modules)

const int RELAY_1 = 2;   // Device 1 – Light / Bulb

const int RELAY_2 = 3;   // Device 2 – Fan

const int RELAY_3 = 4;   // Device 3 – Socket

const int RELAY_4 = 5;   // Device 4 – Spare

// Relay state tracking

bool relay1State = false;

bool relay2State = false;

bool relay3State = false;

bool relay4State = false;

// ── Helper: relay ON = send LOW (active-low module) ──────────

#define RELAY_ON  LOW

#define RELAY_OFF HIGH

// ── Setup ─────────────────────────────────────────────────────

void setup() {

Serial.begin(9600);

Serial.println(F("Home Automation System Starting..."));

BTSerial.begin(9600);

pinMode(RELAY_1, OUTPUT); digitalWrite(RELAY_1, RELAY_OFF);

pinMode(RELAY_2, OUTPUT); digitalWrite(RELAY_2, RELAY_OFF);

pinMode(RELAY_3, OUTPUT); digitalWrite(RELAY_3, RELAY_OFF);

pinMode(RELAY_4, OUTPUT); digitalWrite(RELAY_4, RELAY_OFF);

Serial.println(F("Ready. Waiting for Bluetooth commands..."));

BTSerial.println(F("HOME AUTOMATION READY"));

BTSerial.println(F("Commands: 1-4=ON, a-d=OFF, A=ALL ON, X=ALL OFF, S=Status"));

}

// ── Main Loop ─────────────────────────────────────────────────

void loop() {

if (BTSerial.available()) {

char cmd = (char) BTSerial.read();

processCommand(cmd);

}

if (Serial.available()) {

BTSerial.write(Serial.read());

}

}

// ── Command Processor ─────────────────────────────────────────

void processCommand(char cmd) {

Serial.print(F("Received command: "));

Serial.println(cmd);

switch (cmd) {

case '1':

setRelay(RELAY_1, true,  &relay1State, "Device 1 (Light)");   break;

case '2':

setRelay(RELAY_2, true,  &relay2State, "Device 2 (Fan)");     break;

case '3':

setRelay(RELAY_3, true,  &relay3State, "Device 3 (Socket)");  break;

case '4':

setRelay(RELAY_4, true,  &relay4State, "Device 4 (Spare)");   break;

case 'a':

setRelay(RELAY_1, false, &relay1State, "Device 1 (Light)");   break;

case 'b':

setRelay(RELAY_2, false, &relay2State, "Device 2 (Fan)");     break;

case 'c':

setRelay(RELAY_3, false, &relay3State, "Device 3 (Socket)");  break;

case 'd':

setRelay(RELAY_4, false, &relay4State, "Device 4 (Spare)");   break;

case 'A':

digitalWrite(RELAY_1, RELAY_ON);  relay1State = true;

digitalWrite(RELAY_2, RELAY_ON);  relay2State = true;

digitalWrite(RELAY_3, RELAY_ON);  relay3State = true;

digitalWrite(RELAY_4, RELAY_ON);  relay4State = true;

respond("ALL DEVICES ON");

break;

case 'X':

digitalWrite(RELAY_1, RELAY_OFF); relay1State = false;

digitalWrite(RELAY_2, RELAY_OFF); relay2State = false;

digitalWrite(RELAY_3, RELAY_OFF); relay3State = false;

digitalWrite(RELAY_4, RELAY_OFF); relay4State = false;

respond("ALL DEVICES OFF");

break;

case 'S':

sendStatus();

break;

default:

respond("Unknown command. Send 1-4, a-d, A, X, or S.");

break;

}

}

// ── Set a single relay ────────────────────────────────────────

void setRelay(int pin, bool turnOn, bool* stateVar, const char* label) {

*stateVar = turnOn;

digitalWrite(pin, turnOn ? RELAY_ON : RELAY_OFF);

String msg = String(label) + (turnOn ? " -> ON" : " -> OFF");

respond(msg);

}

// ── Send text back to phone + Serial Monitor ──────────────────

void respond(String msg) {

BTSerial.println(msg);

Serial.println(msg);

}

// ── Send full device status ───────────────────────────────────

void sendStatus() {

BTSerial.println(F("--- STATUS ---"));

BTSerial.print(F("D1 Light : ")); BTSerial.println(relay1State ? "ON" : "OFF");

BTSerial.print(F("D2 Fan   : ")); BTSerial.println(relay2State ? "ON" : "OFF");

BTSerial.print(F("D3 Socket: ")); BTSerial.println(relay3State ? "ON" : "OFF");

BTSerial.print(F("D4 Spare : ")); BTSerial.println(relay4State ? "ON" : "OFF");

BTSerial.println(F("--------------"));

Serial.println(F("--- STATUS ---"));

Serial.print(F("D1 Light : ")); Serial.println(relay1State ? "ON" : "OFF");

Serial.print(F("D2 Fan   : ")); Serial.println(relay2State ? "ON" : "OFF");

Serial.print(F("D3 Socket: ")); Serial.println(relay3State ? "ON" : "OFF");

Serial.print(F("D4 Spare : ")); Serial.println(relay4State ? "ON" : "OFF");

}