#include <Adafruit_LiquidCrystal.h>

// Includes library to control LCD using I2C (simplifies wiring)

int temp = 0;

// Variable to store calculated temperature value

Adafruit_LiquidCrystal lcd_1(0);

// Create LCD object (I2C address = 0 in Tinkercad default)

void setup()

{

pinMode(A0, INPUT);

// Sets analog pin A0 as input (connected to TMP36 sensor)

Serial.begin(9600);

// Starts serial communication at 9600 baud (for debugging/output)

lcd_1.begin(16, 2);

// Initializes LCD with 16 columns and 2 rows

}

void loop()

{

// Read analog value from TMP36 sensor (0–1023 range)

int rawValue = analogRead(A0);

// Convert analog reading to temperature using mapping

// Breakdown:

// (analogRead(A0) - 20) → small offset correction

// * 3.04 → scaling factor (approx conversion to voltage/temp)

temp = map(((rawValue - 20) * 3.04), 0, 1023, -20, 120);

Serial.println(temp);

// Print temperature to Serial Monitor

// Set cursor to first row, first column

lcd_1.setCursor(0, 0);

lcd_1.print("Temp. Monitr. system");

// Display system title

// Set cursor to second row, first column

lcd_1.setCursor(0, 1);

lcd_1.print("Temp Value=");

// Display label

// Move cursor to position where value will be shown

lcd_1.setCursor(12, 1);

lcd_1.print(temp);

// Print temperature value on LCD

delay(10);

// Small delay to stabilize readings and avoid flickering

}